Dominique Anguiano

CPE301 – SPRING 2016

# Design Assignment 6

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | LIBRARY USED IN TASK 1/A |  |  |
| 3. | SCHEMATIC |  |  |
| 4. | FLOW CHART |  |  |
| 5. | SCREENSHOT OF EACH DEMO |  |  |
| 6. | VIDEO LINKS OF EACH DEMO |  |  |
| 7. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

- Atmega328p

- Nokia 5110 GLCD Module

- LM34

| 1. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

/*

* Da 6.c

*

* Created: 4/21/2016 6:18:09 PM

* Author : Dominique

*/

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdint.h>				// needed for uint8_t

#include <avr/interrupt.h>

#include "nokia5110.h"

volatile uint32_t mathy;

volatile uint16_t ADCvalue;		// Global variable, set to volatile if used with ISR

volatile char ones, tens;		// Global variable for the characters to be transmit.

void delay1s();

int main(void)

{

nokia_lcd_init();

nokia_lcd_clear();

nokia_lcd_write_string("Temp in F", 1);

ADMUX = 0;									// use ADC0

ADMUX |= (1 << REFS0);						// use AVcc as the reference

ADCSRA |= (1 << ADPS2) | (1 << ADPS1);		// 64 prescaler for 8Mhz

ADCSRA |= (1 << ADATE);						// Set ADC Auto Trigger Enable

ADCSRB = 0;									// 0 for free running mode

ADCSRA |= (1 << ADEN);						// Enable the ADC

ADCSRA |= (1 << ADIE);						// Enable Interrupts

ADCSRA |= (1 << ADSC);						// Start the ADC conversion

sei();

while(1)

{

}

}

ISR(ADC_vect)

{

ADCvalue = ADC; // only need to read the low value for 8 bit

mathy = ADCvalue;	// Value to perform the conversion between ADC and temperature

mathy = mathy * 5;	// Math to convert from the given ADC value to a temperature value

mathy = mathy * 100;

mathy = mathy / 1024;

ones = mathy % 10;	// Obtaining the ones digit for the temperature value

tens = mathy / 10;	// Obtaining the tens digit for the temperature value

nokia_lcd_set_cursor(0, 10);

nokia_lcd_write_char(tens+'0', 3);	// Output the digits

nokia_lcd_write_char(ones+'0', 3);	// Output the digits

nokia_lcd_write_string(" :)", 3);

nokia_lcd_render();

delay1s();	// Delay for 1 second before converting the next value

}

void delay1s(){

for(int i = 500; i > 0; i--)

_delay_ms(2);

}

| 2. | LIBRARIES USED FOR TASK 1/A |  |  |
| --- | --- | --- | --- |

/* Nokia 5110 LCD AVR Library

*

* Copyright (C) 2015 Sergey Denisov.

* Written by Sergey Denisov aka LittleBuster (DenisovS21@gmail.com)

*

* This library is free software; you can redistribute it and/or

* modify it under the terms of the GNU General Public Licence

* as published by the Free Software Foundation; either version 3

* of the Licence, or (at your option) any later version.

*

* Original library written by SkewPL, http://skew.tk

*/

#include "nokia5110.h"

#include <avr/pgmspace.h>

#include <avr/io.h>

#include <util/delay.h>

#include "nokia5110_chars.h"

static struct {

/* screen byte massive */

uint8_t screen[504];

/* cursor position */

uint8_t cursor_x;

uint8_t cursor_y;

} nokia_lcd = {

.cursor_x = 0,

.cursor_y = 0

};

/**

* Sending data to LCD

* @bytes: data

* @is_data: transfer mode: 1 - data; 0 - command;

*/

static void write(uint8_t bytes, uint8_t is_data)

{

register uint8_t i;

/* Enable controller */

PORT_LCD &= ~(1 << LCD_SCE);

/* We are sending data */

if (is_data)

PORT_LCD |= (1 << LCD_DC);

/* We are sending commands */

else

PORT_LCD &= ~(1 << LCD_DC);

/* Send bytes */

for (i = 0; i < 8; i++) {

/* Set data pin to byte state */

if ((bytes >> (7-i)) & 0x01)

PORT_LCD |= (1 << LCD_DIN);

else

PORT_LCD &= ~(1 << LCD_DIN);

/* Blink clock */

PORT_LCD |= (1 << LCD_CLK);

PORT_LCD &= ~(1 << LCD_CLK);

}

/* Disable controller */

PORT_LCD |= (1 << LCD_SCE);

}

static void write_cmd(uint8_t cmd)

{

write(cmd, 0);

}

static void write_data(uint8_t data)

{

write(data, 1);

}

/*

* Public functions

*/

void nokia_lcd_init(void)

{

register unsigned i;

/* Set pins as output */

DDR_LCD |= (1 << LCD_SCE);

DDR_LCD |= (1 << LCD_RST);

DDR_LCD |= (1 << LCD_DC);

DDR_LCD |= (1 << LCD_DIN);

DDR_LCD |= (1 << LCD_CLK);

/* Reset display */

PORT_LCD |= (1 << LCD_RST);

PORT_LCD |= (1 << LCD_SCE);

_delay_ms(10);

PORT_LCD &= ~(1 << LCD_RST);

_delay_ms(70);

PORT_LCD |= (1 << LCD_RST);

/*

* Initialize display

*/

/* Enable controller */

PORT_LCD &= ~(1 << LCD_SCE);

/* -LCD Extended Commands mode- */

write_cmd(0x21);

/* LCD bias mode 1:48 */

write_cmd(0x13);

/* Set temperature coefficient */

write_cmd(0x06);

/* Default VOP (3.06 + 66 * 0.06 = 7V) */

write_cmd(0xC2);

/* Standard Commands mode, powered down */

write_cmd(0x20);

/* LCD in normal mode */

write_cmd(0x09);

/* Clear LCD RAM */

write_cmd(0x80);

write_cmd(LCD_CONTRAST);

for (i = 0; i < 504; i++)

write_data(0x00);

/* Activate LCD */

write_cmd(0x08);

write_cmd(0x0C);

}

void nokia_lcd_clear(void)

{

register unsigned i;

/* Set column and row to 0 */

write_cmd(0x80);

write_cmd(0x40);

/*Cursor too */

nokia_lcd.cursor_x = 0;

nokia_lcd.cursor_y = 0;

/* Clear everything (504 bytes = 84cols * 48 rows / 8 bits) */

for(i = 0;i < 504; i++)

nokia_lcd.screen[i] = 0x00;

}

void nokia_lcd_power(uint8_t on)

{

write_cmd(on ? 0x20 : 0x24);

}

void nokia_lcd_set_pixel(uint8_t x, uint8_t y, uint8_t value)

{

uint8_t *byte = &nokia_lcd.screen[y/8*84+x];

if (value)

*byte |= (1 << (y % 8));

else

*byte &= ~(1 << (y %8 ));

}

void nokia_lcd_write_char(char code, uint8_t scale)

{

register uint8_t x, y;

for (x = 0; x < 5*scale; x++)

for (y = 0; y < 7*scale; y++)

if (pgm_read_byte(&CHARSET[code-32][x/scale]) & (1 << y/scale))

nokia_lcd_set_pixel(nokia_lcd.cursor_x + x, nokia_lcd.cursor_y + y, 1);

else

nokia_lcd_set_pixel(nokia_lcd.cursor_x + x, nokia_lcd.cursor_y + y, 0);

nokia_lcd.cursor_x += 5*scale + 1;

if (nokia_lcd.cursor_x >= 84) {

nokia_lcd.cursor_x = 0;

nokia_lcd.cursor_y += 7*scale + 1;

}

if (nokia_lcd.cursor_y >= 48) {

nokia_lcd.cursor_x = 0;

nokia_lcd.cursor_y = 0;

}

}

void nokia_lcd_write_string(const char *str, uint8_t scale)

{

while(*str)

nokia_lcd_write_char(*str++, scale);

}

void nokia_lcd_set_cursor(uint8_t x, uint8_t y)

{

nokia_lcd.cursor_x = x;

nokia_lcd.cursor_y = y;

}

void nokia_lcd_render(void)

{

register unsigned i;

/* Set column and row to 0 */

write_cmd(0x80);

write_cmd(0x40);

/* Write screen to display */

for (i = 0; i < 504; i++)

write_data(nokia_lcd.screen[i]);

}

/* Nokia 5110 LCD AVR Library

*

* Copyright (C) 2015 Sergey Denisov.

* Written by Sergey Denisov aka LittleBuster (DenisovS21@gmail.com)

*

* This library is free software; you can redistribute it and/or

* modify it under the terms of the GNU General Public Licence

* as published by the Free Software Foundation; either version 3

* of the Licence, or (at your option) any later version.

*

* Original library written by SkewPL, http://skew.tk

*/

#ifndef __NOKIA_5110_H__

#define __NOKIA_5110_H__

#include <avr/pgmspace.h>

#include <stdint.h>

/*

* LCD's port

*/

#define PORT_LCD PORTB

#define DDR_LCD DDRB

/*

* LCD's pins

*/

#define LCD_SCE 0

#define LCD_RST 1

#define LCD_DC 2

#define LCD_DIN 3

#define LCD_CLK 5

#define LCD_CONTRAST 0x40

/*

* Must be called once before any other function, initializes display

*/

void nokia_lcd_init(void);

/*

* Clear screen

*/

void nokia_lcd_clear(void);

/**

* Power of display

* @lcd: lcd nokia struct

* @on: 1 - on; 0 - off;

*/

void nokia_lcd_power(uint8_t on);

/**

* Set single pixel

* @x: horizontal pozition

* @y: vertical position

* @value: show/hide pixel

*/

void nokia_lcd_set_pixel(uint8_t x, uint8_t y, uint8_t value);

/**

* Draw single char with 1-6 scale

* @code: char code

* @scale: size of char

*/

void nokia_lcd_write_char(char code, uint8_t scale);

/**

* Draw string. Example: writeString("abc",3);

* @str: sending string

* @scale: size of text

*/

void nokia_lcd_write_string(const char *str, uint8_t scale);

/**

* Set cursor position

* @x: horizontal position

* @y: vertical position

*/

void nokia_lcd_set_cursor(uint8_t x, uint8_t y);

/*

* Render screen to display

*/

void nokia_lcd_render(void);

#endif

#include <avr/pgmspace.h>

const uint8_t CHARSET[][5] PROGMEM = {

{ 0x00, 0x00, 0x00, 0x00, 0x00 }, // 20 space

{ 0x00, 0x00, 0x5f, 0x00, 0x00 }, // 21 !

{ 0x00, 0x07, 0x00, 0x07, 0x00 }, // 22 "

{ 0x14, 0x7f, 0x14, 0x7f, 0x14 }, // 23 #

{ 0x24, 0x2a, 0x7f, 0x2a, 0x12 }, // 24 $

{ 0x23, 0x13, 0x08, 0x64, 0x62 }, // 25 %

{ 0x36, 0x49, 0x55, 0x22, 0x50 }, // 26 &

{ 0x00, 0x05, 0x03, 0x00, 0x00 }, // 27 '

{ 0x00, 0x1c, 0x22, 0x41, 0x00 }, // 28 (

{ 0x00, 0x41, 0x22, 0x1c, 0x00 }, // 29 )

{ 0x14, 0x08, 0x3e, 0x08, 0x14 }, // 2a *

{ 0x08, 0x08, 0x3e, 0x08, 0x08 }, // 2b +

{ 0x00, 0x50, 0x30, 0x00, 0x00 }, // 2c ,

{ 0x08, 0x08, 0x08, 0x08, 0x08 }, // 2d -

{ 0x00, 0x60, 0x60, 0x00, 0x00 }, // 2e .

{ 0x20, 0x10, 0x08, 0x04, 0x02 }, // 2f /

{ 0x3e, 0x51, 0x49, 0x45, 0x3e }, // 30 0

{ 0x00, 0x42, 0x7f, 0x40, 0x00 }, // 31 1

{ 0x42, 0x61, 0x51, 0x49, 0x46 }, // 32 2

{ 0x21, 0x41, 0x45, 0x4b, 0x31 }, // 33 3

{ 0x18, 0x14, 0x12, 0x7f, 0x10 }, // 34 4

{ 0x27, 0x45, 0x45, 0x45, 0x39 }, // 35 5

{ 0x3c, 0x4a, 0x49, 0x49, 0x30 }, // 36 6

{ 0x01, 0x71, 0x09, 0x05, 0x03 }, // 37 7

{ 0x36, 0x49, 0x49, 0x49, 0x36 }, // 38 8

{ 0x06, 0x49, 0x49, 0x29, 0x1e }, // 39 9

{ 0x00, 0x36, 0x36, 0x00, 0x00 }, // 3a :

{ 0x00, 0x56, 0x36, 0x00, 0x00 }, // 3b ;

{ 0x08, 0x14, 0x22, 0x41, 0x00 }, // 3c <

{ 0x14, 0x14, 0x14, 0x14, 0x14 }, // 3d =

{ 0x00, 0x41, 0x22, 0x14, 0x08 }, // 3e >

{ 0x02, 0x01, 0x51, 0x09, 0x06 }, // 3f ?

{ 0x32, 0x49, 0x79, 0x41, 0x3e }, // 40 @

{ 0x7e, 0x11, 0x11, 0x11, 0x7e }, // 41 A

{ 0x7f, 0x49, 0x49, 0x49, 0x36 }, // 42 B

{ 0x3e, 0x41, 0x41, 0x41, 0x22 }, // 43 C

{ 0x7f, 0x41, 0x41, 0x22, 0x1c }, // 44 D

{ 0x7f, 0x49, 0x49, 0x49, 0x41 }, // 45 E

{ 0x7f, 0x09, 0x09, 0x09, 0x01 }, // 46 F

{ 0x3e, 0x41, 0x49, 0x49, 0x7a }, // 47 G

{ 0x7f, 0x08, 0x08, 0x08, 0x7f }, // 48 H

{ 0x00, 0x41, 0x7f, 0x41, 0x00 }, // 49 I

{ 0x20, 0x40, 0x41, 0x3f, 0x01 }, // 4a J

{ 0x7f, 0x08, 0x14, 0x22, 0x41 }, // 4b K

{ 0x7f, 0x40, 0x40, 0x40, 0x40 }, // 4c L

{ 0x7f, 0x02, 0x0c, 0x02, 0x7f }, // 4d M

{ 0x7f, 0x04, 0x08, 0x10, 0x7f }, // 4e N

{ 0x3e, 0x41, 0x41, 0x41, 0x3e }, // 4f O

{ 0x7f, 0x09, 0x09, 0x09, 0x06 }, // 50 P

{ 0x3e, 0x41, 0x51, 0x21, 0x5e }, // 51 Q

{ 0x7f, 0x09, 0x19, 0x29, 0x46 }, // 52 R

{ 0x46, 0x49, 0x49, 0x49, 0x31 }, // 53 S

{ 0x01, 0x01, 0x7f, 0x01, 0x01 }, // 54 T

{ 0x3f, 0x40, 0x40, 0x40, 0x3f }, // 55 U

{ 0x1f, 0x20, 0x40, 0x20, 0x1f }, // 56 V

{ 0x3f, 0x40, 0x38, 0x40, 0x3f }, // 57 W

{ 0x63, 0x14, 0x08, 0x14, 0x63 }, // 58 X

{ 0x07, 0x08, 0x70, 0x08, 0x07 }, // 59 Y

{ 0x61, 0x51, 0x49, 0x45, 0x43 }, // 5a Z

{ 0x00, 0x7f, 0x41, 0x41, 0x00 }, // 5b [

{ 0x02, 0x04, 0x08, 0x10, 0x20 }, // 5c backslash

{ 0x00, 0x41, 0x41, 0x7f, 0x00 }, // 5d ]

{ 0x04, 0x02, 0x01, 0x02, 0x04 }, // 5e ^

{ 0x40, 0x40, 0x40, 0x40, 0x40 }, // 5f _

{ 0x00, 0x01, 0x02, 0x04, 0x00 }, // 60 `

{ 0x20, 0x54, 0x54, 0x54, 0x78 }, // 61 a

{ 0x7f, 0x48, 0x44, 0x44, 0x38 }, // 62 b

{ 0x38, 0x44, 0x44, 0x44, 0x20 }, // 63 c

{ 0x38, 0x44, 0x44, 0x48, 0x7f }, // 64 d

{ 0x38, 0x54, 0x54, 0x54, 0x18 }, // 65 e

{ 0x08, 0x7e, 0x09, 0x01, 0x02 }, // 66 f

{ 0x0c, 0x52, 0x52, 0x52, 0x3e }, // 67 g

{ 0x7f, 0x08, 0x04, 0x04, 0x78 }, // 68 h

{ 0x00, 0x44, 0x7d, 0x40, 0x00 }, // 69 i

{ 0x20, 0x40, 0x44, 0x3d, 0x00 }, // 6a j

{ 0x7f, 0x10, 0x28, 0x44, 0x00 }, // 6b k

{ 0x00, 0x41, 0x7f, 0x40, 0x00 }, // 6c l

{ 0x7c, 0x04, 0x18, 0x04, 0x78 }, // 6d m

{ 0x7c, 0x08, 0x04, 0x04, 0x78 }, // 6e n

{ 0x38, 0x44, 0x44, 0x44, 0x38 }, // 6f o

{ 0x7c, 0x14, 0x14, 0x14, 0x08 }, // 70 p

{ 0x08, 0x14, 0x14, 0x18, 0x7c }, // 71 q

{ 0x7c, 0x08, 0x04, 0x04, 0x08 }, // 72 r

{ 0x48, 0x54, 0x54, 0x54, 0x20 }, // 73 s

{ 0x04, 0x3f, 0x44, 0x40, 0x20 }, // 74 t

{ 0x3c, 0x40, 0x40, 0x20, 0x7c }, // 75 u

{ 0x1c, 0x20, 0x40, 0x20, 0x1c }, // 76 v

{ 0x3c, 0x40, 0x30, 0x40, 0x3c }, // 77 w

{ 0x44, 0x28, 0x10, 0x28, 0x44 }, // 78 x

{ 0x0c, 0x50, 0x50, 0x50, 0x3c }, // 79 y

{ 0x44, 0x64, 0x54, 0x4c, 0x44 }, // 7a z

{ 0x00, 0x08, 0x36, 0x41, 0x00 }, // 7b {

{ 0x00, 0x00, 0x7f, 0x00, 0x00 }, // 7c |

{ 0x00, 0x41, 0x36, 0x08, 0x00 }, // 7d }

{ 0x10, 0x08, 0x08, 0x10, 0x08 }, // 7e ~

{ 0x00, 0x00, 0x00, 0x00, 0x00 } // 7f

};

| 3. | SCHEMATICS |  |  |  |
| --- | --- | --- | --- | --- |

| 4. | FLOW CHART |  |  |
| --- | --- | --- | --- |

Initialize the ADC and Nokia LCD

Check if ADC Interrupt Flag is set

Wait

Not Set

Yes, it is set

Head to Interrupt and convert ADC value to temperature.

Output the temperature to the Nokia LCD.

Wait 1 second and exit interrupt

| 5. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

TASK 1/A: Nokia LCD connected to Atmega328p

| 6. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| https://www.youtube.com/watch?v=bT6w2a4MR8s |  |  |  |
| 7. | GOOGLECODE LINK OF THE DA |  |  |
| https://github.com/Anguian3/anguian3-submissions |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Dominique Anguiano